Rover Software

Maze explorer — User Manual

Software version 1.1, 17 October 2026.

For teams using a LEGO SPIKE Prime hub with the SPIKE App 3 (Python).

## Contents

1.  About your package

2.  Your first hour

3.  Your program file

4.  Describing your rover: the SETTINGS

5.  setup_check: testing your rover

6.  Writing your own program

7.  Reacting to colours

8.  The maze explorer

9.  Will the software work with my rover?

10.  Fine tuning

11.  Troubleshooting

12.  Quick reference

# 1. About your package

This is the user manual for the Maze explorer package of the rover software, version 1.1. Everything in the Basic functions package, plus the maze explorer: the rover draws a map as it goes and keeps heading for the nearest place it has not searched yet, until it has searched everywhere it can reach. It can find colours in the order your course needs.

## What you need

- A rover built with a LEGO SPIKE Prime hub, with two drive motors (one driving the left side, one the right) so it can turn on the spot.

- A distance sensor pointing forwards and a colour sensor pointing down at the floor.

- The SPIKE App (version 3), set to Python.

- Your program file: the .py file you were given, with your team name at the top.

- The rover simulator and its manual, to test on a computer before you test on the floor.

## The parts of the file you change

Your program file is long, but you only ever change three parts of it. Everything else is the rover software: read it if you are curious, but do not change it.

| Part | Where | What it is for |
| --- | --- | --- |
| SETTINGS | Near the top | Describe your rover (ports, wheel size, where the sensors are) and your course. |
| YOUR PROGRAM | At the very bottom | Choose what runs, or write your own list of moves. |
| on_colour_found() | In the middle: search for ADD YOUR OWN ACTIONS HERE | What the rover does when it finds a colour. Only if you want your own colour actions. |

Remember: Chapter 2 is a checklist for your first hour. Do it in order, and you will have a working rover before you start on anything clever.

# 2. Your first hour

Work through these steps in order. Each one builds on the one before.

## Step 1: Keep one master copy of your file

The simulator reads your program from a file on the computer; the hub gets it from the SPIKE App. Keep one master copy: the .py file. Change it in a text editor (Notepad, IDLE, VS Code or similar), test it in the simulator, then copy the whole file into the SPIKE App to run it on the rover.

Watch out: Whenever you change the program in the SPIKE App instead, copy it back into the master file straight away, or your team will end up with two different versions.

## Step 2: Put the file into the SPIKE App

- Open the SPIKE App and start a new project. Choose Python.

- Delete the few lines the new project starts with.

- Open your .py file in a text editor, select everything (Ctrl+A), copy it (Ctrl+C) and paste it into the SPIKE App (Ctrl+V).

- Connect the hub and press Run. Messages from the program appear in the SPIKE App’s console.

## Step 3: Fill in the SETTINGS for your rover

Measure your rover and fill in the settings described in chapter 4. Get the ports right first; the measurements can be rough for now, because the tests in step 5 fine-tune them.

## Step 4: Describe your rover in the simulator

Open the simulator, load your file and draw your rover on its Rover tab (see the simulator manual). Its Your program and this rover box tells you if any setting in your file does not match the rover you drew.

## Step 5: Run setup_check on the real rover

At the bottom of the file, make the setup_check line the only one without a # in front, and run it. It tests every part of the rover, one at a time, and tells you which setting to change if something is wrong. Chapter 5 explains each test.

# runloop.run(run_program(my_program))

runloop.run(run_program(setup_check))

# runloop.run(run_program(explore))

## Step 6: Run a program

Put the # back in front of setup_check and take it away from explore. Try it in the simulator first, then on the floor. Chapter 8 explains what it does and how to get the most out of it.

# 3. Your program file

## How the file is laid out

| Section | What is in it |
| --- | --- |
| Header | Your team name, the package and its version. The first line the rover prints is === ROVER SOFTWARE 1.1 ===. |
| Compass directions | NORTH, WEST, SOUTH, EAST and the four in between. You use these names in your program. |
| SETTINGS | The settings every team must fill in (chapter 4). |
| FINE TUNING | Speeds and other numbers you can normally leave alone (chapter 10). |
| The rover software | One section per part: Setup, Movement, Distance sensor, Colour sensor, Grid moves, Maze explorer. Each section starts with a short description of what it does and which functions you can use. |
| YOUR PROGRAM | Your own program, and the lines that choose what runs. |

## Choosing what runs

At the very bottom there is one runloop.run(...) line for each program. Exactly one of them must have no # in front: that is the one that runs.

# Choose what runs: exactly ONE of these lines must have no # in front.

# runloop.run(run_program(my_program))

# runloop.run(run_program(setup_check))

runloop.run(run_program(explore))

run_program does three things before your program starts: it checks every setting, sets the gyro’s zero (the way the rover faces now becomes NORTH), and starts the clock for MAX_RUN_SECONDS. So place the rover facing the way you want to call NORTH before you press Run.

## Messages from the rover

Everything the program prints appears in the SPIKE App’s console (and in the simulator’s Log). The rover software prints a line for each move, such as drive: done after 30.0 cm or turn: to 90, stopped at 89.4. Add your own with print(...):

print("Reached the bridge")

print("Distance ahead: " + str(read_distance()) + " cm")

Tip: If the program finds a mistake in your settings it prints SETTINGS PROBLEM: and a message saying which setting is wrong and what it should be, and the rover does not move. Fix the setting and run again.

## Stopping

Press the hub’s middle button to stop the program at any time. Inside your own program, end_run() stops the whole run from anywhere.

# 4. Describing your rover: the SETTINGS

The SETTINGS tell the software how your rover is built. Every team’s rover is different, so every team must fill them in. A wrong setting makes the rover drive strangely, so take your time here.

## The turning point

Several settings are measured from the rover’s turning point: the spot that stays still when the rover spins on the spot.

- With two drive wheels and a caster (or ball), the turning point is half way between the two drive wheels.

- With four or more wheels that cannot swivel, it is somewhere between them. setup_check test 6 finds it (chapter 5).

Measure forwards and backwards along the middle of the rover, in centimetres. In front of the turning point is plus, behind it is minus.

## Your rover

| Setting | What it means | How to find it |
| --- | --- | --- |
| LEFT_MOTOR_PORT RIGHT_MOTOR_PORT | The hub port of the motor driving the left wheels and the right wheels: port.A to port.F. | Look at the hub. Tests 1 and 2 tell you if they are swapped. |
| DISTANCE_SENSOR_PORT COLOUR_SENSOR_PORT | The ports of the two sensors. | Look at the hub. Tests 7 and 8 check them. |
| LEFT_MOTOR_DIRECTION RIGHT_MOTOR_DIRECTION | 1 or -1: which way each motor must spin to roll the rover forwards. Motors mounted mirror-image need opposite numbers. | Tests 1 and 2. |
| WHEEL_DIAMETER_CM | The diameter of the drive wheels, across the tyre. | Measure with a ruler; test 4 fine-tunes it. |
| WHEEL_GEAR_RATIO | How many times the motor turns for one turn of the wheel. 1 if the wheel is fixed straight onto the motor. With gears: teeth on the wheel’s gear ÷ teeth on the motor’s gear. | Count the teeth. |
| ROVER_WIDTH_CM | The width of the rover at its widest point. | Ruler. |
| ROVER_FRONT_CM | From the turning point to the part that sticks out furthest at the front. | Test 6. |
| ROVER_BACK_CM | From the turning point to the part that sticks out furthest at the back. | Ruler, from the turning point. |
| DISTANCE_SENSOR_POSITION_CM | Where the front of the distance sensor is, measured from the turning point. Plus = in front. | Test 7 suggests the number. |
| COLOUR_SENSOR_POSITION_CM | Where the colour sensor is, measured from the turning point. Plus = in front, minus = behind. | Ruler. |
| HUB_FACE_UP | Which side of the hub faces up. motion_sensor.TOP when the hub lies flat with its light matrix facing up. Others: BOTTOM, FRONT, BACK, LEFT, RIGHT. | Test 3. |
| MAX_RUN_SECONDS | End the run after this many seconds. 0 = no limit. | Your competition rules. |

## Colours

| Setting | What it means |
| --- | --- |
| COLOURS_TO_FIND | The colours the rover reacts to, for example [color.GREEN, color.RED]. The names are color.RED, color.GREEN, color.BLUE, color.YELLOW and so on (the SPIKE spelling, without a u). |
| COLOUR_PAUSE_MS | How long the rover stops on a colour, in milliseconds. 5000 = 5 seconds. |
| REACT_ONCE | True: react to each colour only the first time. False: react every time the rover drives onto it. |

## Your course

| Setting | What it means |
| --- | --- |
| COURSE_NORTH_CM COURSE_EAST_CM COURSE_WEST_CM COURSE_SOUTH_CM | How far the course reaches from the start, in each direction. The rover never goes outside these limits. NORTH is the way the rover faces at the start, EAST to its right, WEST to its left. Use 0 for SOUTH when the start is at the edge of the course, so the rover cannot wander out through the entrance. Keep them close to your real course: very large limits make the planning slow. |
| CHECK_BY_WALLS | True: also check the strip of floor next to each wall the rover meets, which its normal path never reaches. Slower, but finds patches against walls. |
| HEAD_TOWARDS | Which way to search first, as a heading in degrees (0 = NORTH, 90 = WEST, -90 = EAST), or None for no preference. See chapter 8. |

Tip: The Copy settings for my program button on the simulator’s Rover tab writes the rover settings for you, from the rover you drew.

# 5. setup_check: testing your rover

setup_check tests each part of the rover, one at a time, and tells you which setting to change if something is wrong. Run it on every new rover, and again after you rebuild anything.

## How to use it

- Make runloop.run(run_program(setup_check)) the only runloop.run line without a #, and run the program.

- Each test prints what it is about to do and waits. Press the hub’s RIGHT button to start it.

- When a test has finished, press RIGHT for the next test, or LEFT to do the same test again (for example to measure twice).

- Read the console in the SPIKE App: each test says what to look for and which setting to change.

- A changed setting only counts when you run the program again.

- You need about a metre of clear floor in front of the rover.

## The eight tests

| Test | What happens | If it is wrong |
| --- | --- | --- |
| 1. Left wheel | Only the left wheel turns for 1 second, rolling forwards. | The right wheel turned: swap LEFT_MOTOR_PORT and RIGHT_MOTOR_PORT. It rolled backwards: change LEFT_MOTOR_DIRECTION (1 ↔ -1). |
| 2. Right wheel | Only the right wheel turns, rolling forwards. | It rolled backwards: change RIGHT_MOTOR_DIRECTION. |
| 3. Gyro | The rover spins left for 1 second and prints the degrees the gyro measured. It should be a plus number. | It measured a right turn: if the rover really turned right, swap the motor ports; if it turned left, the hub is upside down, so change HUB_FACE_UP. Hardly any change: the hub is on its side, so change HUB_FACE_UP. |
| 4. Drive 30 cm | Mark the floor beside the middle of a wheel. The rover drives 30 cm. | Measure how far the mark beside the wheel really moved. If it was not 30 cm, the new WHEEL_DIAMETER_CM = the old value × (cm it really moved) ÷ 30. The test prints this sum for you. |
| 5. Turning | A quarter turn left, then back. The turn: lines should stop within 2 degrees of the target. It also prints the motor degrees for a quarter turn. | Turns wobble or overshoot a lot: lower TURN_SLOW_SPEED (chapter 10). Type the motor degrees into the simulator (Rover tab, Match my real rover). |
| 6. Turning point | Mark the floor at the very front of the rover, in the middle. The rover turns half a turn. Mark the front again. | The turning point is half way between the two marks, so ROVER_FRONT_CM is half the distance between them. Measure ROVER_BACK_CM and the sensor positions from that point too. |
| 7. Distance sensor | Live readings: move your hand in front of the sensor. Press RIGHT, then stand the rover facing a wall with its turning point exactly 30 cm from the wall, and press RIGHT again. | It prints the DISTANCE_SENSOR_POSITION_CM to use. No echo: point the sensor straight at the wall. A suggestion very different from your setting usually means the 30 cm was measured to the sensor instead of the turning point. |
| 8. Colour sensor | Live readings: slide coloured paper under the sensor and watch the colour names. | Wrong or no colours: check COLOUR_SENSOR_PORT, and that the sensor faces the floor and sits close to it. |

# 6. Writing your own program

Your own program goes in my_program(), in the YOUR PROGRAM section at the bottom of the file. It is a list of moves, done one after the other.

async def my_program():

await drive_forward(50)

await turn_left(90)

await drive_forward(30)

## Two rules

- Every move needs await in front: await drive_forward(50), not drive_forward(50). Without await the program does not wait for the move and goes straight on to the next line. The functions that need it are marked with await in the tables below.

- Indent every line of your program by four spaces, so Python knows it belongs to my_program().

## Directions

NORTH is the way the rover faced when the program started. The other directions are fixed to the room, not to the rover: WEST is always 90 degrees to the left of the start direction, whichever way the rover faces now.

| Name | Heading | Which way |
| --- | --- | --- |
| NORTH | 0 | The way the rover faced at the start |
| WEST | 90 | A quarter turn left of that |
| SOUTH | 180 | Back the way it came |
| EAST | -90 | A quarter turn right of the start |
| NORTH_WEST, SOUTH_WEST, SOUTH_EAST, NORTH_EAST | 45, 135, -135, -45 | Half way between |

Headings are in degrees, and turning left makes the number bigger. Any number works, so turn_to(30) faces a little left of NORTH.

Tip: turn_to(WEST) is more accurate than turn_left(90) over a long route: it always turns to the exact direction, so small errors do not add up.

## Moving

| Function | What it does |
| --- | --- |
| await drive_forward(cm) | Drive forwards in a straight line. The gyro keeps it straight. |
| await drive_forward(cm, stop_at_wall_cm=10) | The same, but stop early if a wall comes closer than 10 cm. |
| await drive_backward(cm) | Drive backwards. There is no sensor at the back, so make sure there is room. |
| await turn_left(degrees) await turn_right(degrees) | Turn on the spot by up to 180 degrees. Without a number, 90. |
| await turn_to(direction) | Turn on the spot to face a direction: NORTH, WEST, … or any heading. |
| cm_driven() | How far the last drive really went, in cm. |
| rover_position() | Where the rover is: (cm to the right of the start, cm ahead of the start). Minus means left of, or behind, the start. |
| gyro_heading() | Which way the rover faces now, in degrees. |

## The distance sensor

| Function | What it does |
| --- | --- |
| read_distance() | The distance to whatever is ahead, in cm, right now. -1 means no echo: nothing in range, or the sound bounced away. |
| await measure_distance() | A careful reading, standing still: it takes three readings and uses the middle one. Use it for decisions. |
| await drive_to_wall(stop_cm) | Drive forwards until something is stop_cm away (at most 2 m). |

## Colours and the run

| Function | What it does |
| --- | --- |
| read_colour() | The colour under the sensor right now, such as color.GREEN. |
| colour_name(colour) | The name of a colour as text, for printing: colour_name(color.GREEN) gives "GREEN". |
| found_colours | A list of the colours found so far. |
| end_run() | Finish the whole run straight away. |

## What a move tells you

Drives give back how they ended, and turns give back whether they worked. You can use that to decide what to do next:

| Move | Gives back |
| --- | --- |
| drives | "done": drove the whole way. "wall": stopped because a wall came closer than stop_at_wall_cm. "stuck": the wheels stopped turning, because the rover is pushing against something. |
| turns | True: it now faces the new way. False: there was not enough room for its back to swing round (it tried the other way round, and shuffling sideways, first). It then still faces the way it did before. |

async def my_program():

result = await drive_forward(120, stop_at_wall_cm=12)

if result == "wall":

print("Wall after " + str(cm_driven()) + " cm")

await turn_to(WEST)

await drive_to_wall(12)

## Examples

### A route with compass directions

async def my_program():

await drive_forward(60)            # 60 cm north

await turn_to(WEST)

await drive_to_wall(15)            # on until 15 cm from the wall

await turn_to(NORTH)

await drive_forward(80, stop_at_wall_cm=10)

### Doing something several times

async def my_program():

for side in range(4):              # a square: four sides

await drive_forward(30)

await turn_left(90)

### Driving to a good starting place, then going autonomous

async def my_program():

await drive_forward(40)

await turn_to(WEST)

await explore()

Choose my_program at the bottom of the file to run this.

Tip: Plan a route with walls, not just distances: drive_to_wall() and stop_at_wall_cm correct small errors, because the wall is always where it is. A route made only of distances drifts a little further off with every move.

# 7. Reacting to colours

The colour sensor watches the floor during every drive and turn, including the autonomous ones, without you doing anything. When it arrives on a colour from COLOURS_TO_FIND, the rover stops and on_colour_found() runs. Afterwards the move it was making carries on.

Out of the box, on_colour_found() prints the colour and pauses for COLOUR_PAUSE_MS (5 seconds). To add your own actions, find it in the file: search for ADD YOUR OWN ACTIONS HERE. Change the lines after the big comment, for example:

async def on_colour_found(colour):

# (the big comment stays here)

print("COLOUR: found " + colour_name(colour))

if colour == color.GREEN:

await motor.run_for_degrees(port.E, 180, 300)   # tip the payload out

await runloop.sleep_ms(1000)

if colour == color.RED:

end_run()                                       # finish the run here

Watch out: Do not drive the rover inside on_colour_found(). When it finishes, the move that was interrupted carries on, and the autonomy still thinks the rover is where it stopped. Other motors (like a payload arm on port E), the lights and sounds are fine.

- The sensor reacts when it arrives on a colour, so sitting on a patch only counts once.

- With REACT_ONCE = True, each colour only counts the first time it is found.

- found_colours lists the colours found so far, so if len(found_colours) == len(COLOURS_TO_FIND): end_run() finishes once every colour has been found.

Remember: Need to find colours in a set order, or drive as part of your colour actions? Use explore(find=...) instead (chapter 8). on_colour_found() is then switched off.

# 8. The maze explorer

The explorer draws a map of the course as it goes. It keeps heading for the nearest square it has not searched yet, until it has searched everywhere it can reach. It finds patches out in the open that a wall follower would miss, but it takes longer.

## Two ways to use it

| Call | What it does |
| --- | --- |
| await explore() | Search everywhere. on_colour_found() reacts to colours on the way (chapter 7). Choose explore at the bottom of the file to run just this. |
| await explore(find=color.GREEN) | Search until the colour sensor is on GREEN, stop there, and carry on with the next line of your program. Gives back True if it found it, or False if it searched everywhere it can reach (twice) without finding it. |

## Finding colours in order

Write one explore(find=...) for each colour, in the order they must be found, in my_program(). The lines after each one are your actions for that colour, and they may drive:

async def my_program():

if not await explore(find=color.GREEN, near=(-30, 110)):

return                    # no GREEN anywhere: stop here

await drive_forward(8)        # e.g. get the payload over the patch

# ...your payload code here...

await explore(find=color.RED, towards=30)

- Only the colour it is looking for counts: driving over RED while it looks for GREEN does nothing. (on_colour_found() is switched off once you use find.)

- It keeps its map from one explore() to the next, so it never searches the same floor twice. If it drove over a colour before that colour’s turn, it goes straight back there.

- It keeps track of how far your own moves drive it, and starts the next explore() from the middle of the nearest square.

## Telling it where to search first

Both hints are optional. The rover still searches everywhere in the end; it just searches there first.

| Hint | What it means |
| --- | --- |
| near=(-30, 110) | Roughly where the colour is: (cm to the right of the start, cm ahead of it). Use a minus for left or behind: (-30, 110) is 30 cm left and 110 cm ahead. The rover heads there, then searches around it. |
| towards=30 | Which way to search, as a heading in degrees. The rover keeps going that way until walls stop it. |
| neither | It uses the HEAD_TOWARDS setting (None = just the nearest unsearched square). |

Tip: Use a heading between two directions, like 30 (“mostly north, a bit west”), rather than exactly NORTH_WEST (45). At 45 north and west count exactly the same, so the rover cannot choose and just keeps going the way it faces, which can lead it into a dead end.

## How it works

The rover thinks of the floor as a grid of squares, STEP_CM (12 cm) wide. It starts in square (0, 0) facing NORTH; the first number counts squares to the east (right), the second squares to the north (ahead). Its map remembers which squares it has stood on, which moves between squares are open or blocked by a wall, and which squares are too close to a wall to ever stand in. Each time round, it:

- Plans: picks the unsearched square to head for, with a “ripple search” that spreads out from the rover square by square, so it always finds the shortest route.

- Looks: if it does not know whether the first move of that route is open, it faces that way and measures. One look maps a whole line of squares.

- Moves one square, or marks that move as blocked if it could not.

## Reading its messages

| Message | Meaning |
| --- | --- |
| explore: heading for square (-3, 8) | Its next target: 3 squares west and 8 north of the start. |
| look: WEST from (-1, 3), 81.7 cm, 5 squares free | It measured 81.7 cm to the west and marked 5 squares open. |
| step: NORTH blocked (wall), going back | A wall stopped it; it marked that move as blocked. |
| map ¦ ... | A picture of its map, NORTH at the top. R = rover, o = visited, . = seen to be open, # = too close to a wall to stand in. |
| explore: searched every square it can reach - finished | It has searched everywhere it can reach. |

## What it cannot do

- It is slower than a wall follower: a full search of a 2 m course can take several minutes. Use hints, and set MAX_RUN_SECONDS to your competition time.

- It cannot reach places through a gap narrower than the circle the rover needs to turn in (see chapter 9).

- It moves in 12 cm squares, so a patch smaller than about that can lie between its paths. CHECK_BY_WALLS covers the strips next to walls, except the last 5 cm (WALL_CHECK_GAP_CM).

- It trusts its map. If the rover is pushed, or slips a lot, the map no longer matches the course.

# 9. Will the software work with my rover?

The software works with most rover designs, but not all. Check yours before you build too much.

## It must turn on the spot

The software turns the rover by running the left wheels backwards and the right wheels forwards (or the other way). That needs driven wheels on both sides. Rovers that steer like a car, with a steering motor, cannot turn on the spot, so the turns, the wall follower and the explorer do not work with them.

## Room to turn

When the rover spins on the spot, its corners sweep a circle round the turning point. It needs a clear circle twice as wide as the distance from the turning point to its furthest corner. A rover 15 cm wide with 11 cm behind the turning point needs about 27 cm. The simulator’s Rover tab shows the number for your rover.

- If a turn has no room, the rover swings back and tries the long way round, then shuffles a few centimetres sideways and tries again. If nothing works, the turn gives back False.

- A compact rover, with the turning point near the middle, gets through more of the course.

## Different wheel layouts

| Layout | Does it work? |
| --- | --- |
| Two drive wheels and a caster or ball | Yes: the simplest and most accurate. |
| Four or six wheels, the wheels on each side geared to one motor (skid steer) | Yes. The wheels slide sideways when it turns, so turns take more motor degrees and the turning point is between the axles. Run tests 5 and 6 and enter the results in the simulator. |
| Two drive wheels at the back, two free wheels at the front that cannot swivel | Works, but it turns round the front axle and needs a lot of room. A caster or ball at the front is much better. |
| Driven wheels of different sizes on the same side | No: they roll at different speeds, fight each other and slip. |
| Car steering | No (see above). |

## Two motors on each side

The software drives one motor per side. If your rover has a second motor on each side, you need to change three functions in the Setup section: set_wheel_speeds, stop and reset_wheel_counters. Add one line for each extra motor, like this (the extra motors here are on ports E and F):

def set_wheel_speeds(left, right):

motor.run(LEFT_MOTOR_PORT, int(left * LEFT_MOTOR_DIRECTION))

motor.run(RIGHT_MOTOR_PORT, int(right * RIGHT_MOTOR_DIRECTION))

motor.run(port.E, int(left * LEFT_MOTOR_DIRECTION))      # second left motor

motor.run(port.F, int(right * RIGHT_MOTOR_DIRECTION))    # second right motor

def stop():

motor.stop(LEFT_MOTOR_PORT)

motor.stop(RIGHT_MOTOR_PORT)

motor.stop(port.E)

motor.stop(port.F)

def reset_wheel_counters():

motor.reset_relative_position(LEFT_MOTOR_PORT, 0)

motor.reset_relative_position(RIGHT_MOTOR_PORT, 0)

motor.reset_relative_position(port.E, 0)

motor.reset_relative_position(port.F, 0)

Watch out: This assumes each extra motor is mounted the same way round as the first motor on its side. If it is mounted the other way round, use a minus: int(-left * LEFT_MOTOR_DIRECTION). Check with tests 1 and 2: both motors on a side must roll their wheels forwards.

## Sensors

- The distance sensor must point straight ahead, level, at the height of the walls. On the middle line is best: off to one side, it can see past corners the middle of the rover meets.

- The colour sensor must face the floor, close to it. It can be in front of or behind the wheels; set COLOUR_SENSOR_POSITION_CM to match.

- The hub can sit any way up, as long as HUB_FACE_UP says which way.

# 10. Fine tuning

The FINE TUNING settings are below the SETTINGS in your file. The default values (in brackets) work for most rovers, so leave them alone unless something below describes your problem.

Remember: Change one setting at a time, write down the old value, and try it in the simulator before the real rover.

| Setting (default) | What it does, and when to change it |
| --- | --- |
| DRIVE_SPEED (400) | Wheel speed when driving, in degrees per second (up to about 1000). Lower it if the wheels slip or the rover tips. |
| DRIVE_SLOW_SPEED (150), DRIVE_SLOW_ZONE_CM (3) | The rover slows down for the last few cm of a drive, so it stops on the spot. If drives overshoot, make the zone bigger or the slow speed lower. |
| TURN_SPEED (200), TURN_SLOW_SPEED (100), TURN_SLOW_ZONE (15) | The same for turns, in degrees. If turns overshoot or wobble, lower TURN_SLOW_SPEED. |
| TURN_ACCURACY (2), TURN_NUDGE_SPEED (30) | A turn that ends more than 2 degrees off is corrected with slow nudges. If the rover sits still trying to nudge (a heavy rover, or one with many wheels), raise TURN_NUDGE_SPEED. |
| STEERING (10) | How strongly the gyro steers the rover back when it drifts off course. 0 = off. If it wiggles while driving, lower it. |
| STUCK_MS (1000) | Wheels not moving for this long = stuck. |
| TICK_MS (20) | How often it checks the sensors while moving. Leave it. |
| MAX_TRUSTED_CM (180) | Distance readings further than this count as “no echo”. Set it a little below the longest straight path in your course. |
| SETTLE_MS (150) | How long a careful reading waits for the rover to stop wobbling. |
| STEP_CM (12) | The size of one grid square. Smaller squares fit through narrower gaps and miss fewer small patches, but the search takes longer. |
| WALL_STOP_CM (10) | A step stops early if a wall is this close. |
| WALL_CHECK_GAP_CM (5) | How close the wall check brings the rover to the wall. |
| HEAD_STRENGTH (1) | How strongly the explorer follows a hint: how many extra steps it will drive to reach a square one step further the way you asked. |
| NO_ECHO_SQUARES (2) | When a look gets no echo, how many squares ahead it counts as open. |

# 11. Troubleshooting

| What you see | What to do |
| --- | --- |
| SETTINGS PROBLEM: ... and the rover does not move | A setting is wrong. The message says which, and what it should be. |
| Nothing happens, or the program ends at once | Exactly one runloop.run(...) line at the bottom must have no # in front. |
| A move is skipped, or several happen at once | A move is missing await in front. |
| The wrong wheel turns, or a wheel rolls backwards | setup_check tests 1 and 2. |
| The rover spins and spins, or turns the wrong way | setup_check test 3 (HUB_FACE_UP, or swapped motor ports). |
| Drives are too long or too short | setup_check test 4 (WHEEL_DIAMETER_CM, WHEEL_GEAR_RATIO). |
| Turns overshoot or wobble | Lower TURN_SLOW_SPEED (chapter 10). |
| turn: no room to turn ... | The rover is too close to a wall to spin. Give it more room, or make the rover more compact (chapter 9). |
| drive: stuck after ... | The rover was pushing against something. |
| The distance sensor always says no echo (-1) | Check DISTANCE_SENSOR_PORT and that the sensor points straight at the walls. Walls seen at a slant bounce the sound away. |
| Colours are never found | Check COLOURS_TO_FIND, COLOUR_SENSOR_PORT and setup_check test 8. Is the sensor close to the floor? |
| The run stops after a while | MAX_RUN_SECONDS is set. |
| TypeError: motor speeds must be whole numbers | You used motor.run yourself with a decimal number: use int(...). |
| It works in the simulator but not on the floor | Compare your settings with the simulator’s Rover tab, and run setup_check again on the real rover. |
| The rover wanders, and its map makes no sense | Usually WHEEL_DIAMETER_CM or the rover size is wrong: every step is then the wrong length. Check the settings with the simulator’s Rover tab. |

# 12. Quick reference

## Functions

await drive_forward(cm)                  await drive_forward(cm, stop_at_wall_cm=10)

await drive_backward(cm)                 await drive_to_wall(stop_cm)

await turn_left(degrees)                 await turn_right(degrees)

await turn_to(NORTH / WEST / SOUTH / EAST / any heading)

read_distance()     await measure_distance()     cm_driven()     rover_position()

read_colour()       colour_name(colour)          found_colours   end_run()

await explore()     await explore(find=color.GREEN, near=(x, y))     await explore(find=color.RED, towards=30)

## Directions

NORTH 0 (start direction) · WEST 90 · SOUTH 180 · EAST -90 · turning left makes the number bigger.

## Your program

async def my_program():

await drive_forward(50)        # every move needs await

await turn_to(WEST)

## setup_check

RIGHT = start / next test · LEFT = repeat the test · middle button = stop. Tests: 1 left wheel, 2 right wheel, 3 gyro, 4 drive 30 cm, 5 turning, 6 turning point, 7 distance sensor, 8 colour sensor.